# DESIGNATION  - ASHE

PID  - NONE, SET NEW MONUMENT

N 35 07 46.1

# W 119 04 29.4

Set new monument on concrete bridge abutment.

Additional recovery notes:

Station is located about 17 miles south of Bakersfield and about 3.2 miles northwest of Lakeview at the Ashe Rd. overcrossing of I-5.  Set on north abutment on west side in pedestrian sidewalk.

To reach from the Old River Rd exit on Interstate 5 go south from the intersection on Old River Rd. 0.6 miles to a paved crossroad (Herring Rd).  Turn left (east) on Herring Rd and go 2.1 miles (crossing I-5 at 0.6 miles) to paved crossroad (Ashe Rd.).  Turn right (south) on Ashe Rd. and go 1.6 miles past an old cotton gin processing complex to the I-5 overcorssing (bridge no. 50-294) and the station on the right.

Station is located 14 feet west of the C/L of road, 1 feet south of north end of the abutment and 1.0 feet west of west face of curb.